## Real-Time Audio Noise Reduction Using Fourier Transform Techniques

### A CAPSTONE PROJECT REPORT

A Capstone Project Report submitted in partial fulfilment of the requirements for the Course of

UBA0620-APPLIED MATHEMATICS

to the award of the degree of

### BACHELOR OF ENGINEERING

ELECTRONICS AND COMMUNICATION ENGINEERING

Submitted by

ASHWATH DRAVID(192512175)

Lakshmi Narayanan N T N(192524353)

Ashvin M I J(192572155)

Under the Supervision of

Dr. SUDAR MOZHLI

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences Chennai-602105

SEPTEMBER 2026

# SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences Chennai-602105

## BONAFIDE CERTIFICATE

This is to certify that the Capstone Project entitled Real-Time Audio Noise Reduction Using Fourier Transform Techniques has been carried out by Ashwath dravid under the supervision of Dr. Sudar Mozhli  and is submitted in partial fulfilment of the requirements for the current semester of the B.Tech Electronics and Communication Engineering program at Saveetha Institute of Medical and Technical Sciences, Chennai.

#### COURSE FACULTY

Dr.K. SUDAR MOZHI,
Assistant Professor (SG),
Department of Pure and Applied Mathematics,
SIMATS ENGINEERING, SIMATS
Chennai.
Tamil Nadu.

# SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences Chennai-602105

## DECLARATION

I, Ashwath dravid of the ECE, SIMATS Engineering, Saveetha Institute of Medical and Technical Sciences, Chennai, hereby declare that the Capstone Project Work entitled Real-Time Audio Noise Reduction Using Fourier Transform Techniques is the result of our own bonafide efforts. To the best of our knowledge, the work presented herein is original, accurate, and has been carried out in accordance with the principles of engineering ethics and academic integrity. All sources, references, data, and other materials used in the project have been appropriately acknowledged. We further declare that the data, results, analysis, and findings presented in this report have not been fabricated, falsified, or manipulated. Any external tools, software, datasets, computational resources, or AI-assisted tools used during the project have been appropriately disclosed. We confirm that all applicable ethical, academic, institutional, and professional requirements have been duly followed throughout the planning, execution, analysis, and documentation of the project.

Place:CHENNAI

Date:

Signature of the Students with Names

# SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences Chennai-602105

## Individual Contribution Statement

| Student Name / Register No. | Specific Responsibilities | Design & Development Contribution | Testing & Analysis Contribution | Report Contribution | Approx. Contribution (%) |
| --- | --- | --- | --- | --- | --- |
| Ashwath dravid (192512175) | Audio Input & Signal Acquisition | Audio input setup, microphone interfacing, and acquisition of noisy audio signals | Tested audio input and analyzed the quality of recorded noisy signals | Contributed to hardware setup, audio acquisition, and testing sections | 33% |
| Lakshmi Narayanan N T N (192524353) | Fourier Transform & Noise Reduction Algorithm | Implemented FFT, frequency-domain analysis, filtering, and noise-reduction algorithm | Tested different noise conditions and compared noisy and filtered audio signals | Contributed to methodology, FFT algorithm, signal processing, and results analysis | 33% |
| Ashvin M I J (192572155) | System Integration & Performance Evaluation | Integrated audio acquisition, Fourier processing, filtering, and output generation into the complete system | Evaluated noise reduction performance, audio quality, processing response, and system stability | Contributed to system design, performance evaluation, conclusion, and final documentation | 33% |
| Total |  |  |  |  | 100% |

# ABSTRACT

Audio signals are frequently affected by unwanted background noise originating from environmental conditions, electronic devices, machinery, human activity, and communication channels. Such noise can reduce speech intelligibility, decrease audio quality, and affect the performance of audio-processing and communication systems. The engineering problem addressed in this project is the development of a real-time audio noise reduction system capable of analyzing an input audio signal, identifying unwanted frequency components, reducing the noise, and reconstructing an improved audio signal.

The proposed Real-Time Audio Noise Reduction Using Fourier Transform Techniques uses digital signal processing principles, particularly the Fast Fourier Transform (FFT), to analyze audio signals in the frequency domain. The input audio signal is first acquired through an audio input device and converted into digital samples. The FFT transforms the time-domain signal into its frequency-domain representation, allowing significant frequency components and unwanted noise components to be identified. A suitable frequency-domain filtering technique is then applied to attenuate unwanted frequency components. Finally, the filtered frequency-domain signal is converted back into the time domain using the Inverse Fast Fourier Transform (IFFT).

The system is designed to operate on audio data in real time by processing the signal in small frames. The implementation involves audio acquisition, signal preprocessing, FFT computation, noise identification, frequency-domain filtering, IFFT reconstruction, and audio output. Experimental testing can be performed using audio signals containing different types and levels of background noise. The performance of the system can be evaluated by comparing the original noisy signal with the processed signal using waveform analysis, frequency-spectrum analysis, and suitable audio-quality measures.

The developed system demonstrates how Fourier Transform techniques can be applied to a practical engineering problem in digital audio processing. The project provides a low-cost and computationally practical approach for reducing unwanted noise while preserving useful audio information. The system can be further extended using adaptive filtering, spectral subtraction, Wiener filtering, machine-learning-based noise classification, and advanced real-time digital signal-processing techniques.

Keywords: Audio Noise Reduction, Fourier Transform, Fast Fourier Transform, FFT, IFFT, Digital Signal Processing, Frequency Domain, Audio Filtering, Real-Time Processing.

# TABLE OF CONTENTS

| Sl. No. | Title | Page No. |
| --- | --- | --- |
| i | Certificate from the Supervisor | 1 |
| ii | Declaration by the Candidate | 2 |
| iii | Individual Contribution Statement | 3 |
| iv | Abstract | 4 |
| v | List of Figures | 5 |
| vi | List of Tables | 6 |
| vii | List of Abbreviations / Symbols | 7 |
| 1 | Chapter 1: Introduction and Engineering Problem | 8 |
| 2 | Chapter 2: Literature Review and Existing Solutions | 12 |
| 3 | Chapter 3: Engineering Design & Trade-offs, Sustainability, Ethics, Safety, Risk & Security | 16 |
| 4 | Chapter 4: Implementation, Testing & Results, Project Management | 20 |
| 5 | Chapter 5: Conclusion, Future Work and Reflection | 24 |
| — | References | 26 |
| — | Appendices | 28 |

# LIST OF FIGURES

| Figure No. | Figure Name | Page No. |
| --- | --- | --- |
| Fig. 3.1 | Proposed Real-Time Audio Noise Reduction System Architecture | 15 |
| Fig. 3.2 | Audio Noise Reduction Processing Flowchart | 16 |
| Fig. 3.3 | Time-Domain and Frequency-Domain Representation | 17 |
| Fig. 3.4 | FFT-Based Filtering Process | 18 |
| Fig. 4.1 | Hardware/Software Implementation Setup | 21 |
| Fig. 4.2 | Noisy Audio Waveform | 22 |
| Fig. 4.3 | FFT Spectrum of Noisy Audio | 22 |
| Fig. 4.4 | Filtered Frequency Spectrum | 23 |
| Fig. 4.5 | Reconstructed Denoised Audio Waveform | 23 |
| Fig. 4.6 | Comparison of Noisy and Denoised Audio | 24 |

# LIST OF TABLES

| Table No. | Table Name | Page No. |
| --- | --- | --- |
| Table 2.1 | Review of Existing Audio Noise Reduction Approaches | 13 |
| Table 2.2 | Comparative Analysis of Existing Solutions | 14 |
| Table 3.1 | Stakeholder/User Requirements | 15 |
| Table 3.2 | Functional and Non-Functional Requirements | 16 |
| Table 3.3 | Applicable Standards and Guidelines | 17 |
| Table 3.4 | Design Specifications | 17 |
| Table 3.5 | Alternative Solution Evaluation | 18 |
| Table 3.6 | Design Trade-off Analysis | 19 |
| Table 3.7 | Sustainability, Ethics, Safety and Security Analysis | 19 |
| Table 3.8 | Project Risk Register | 20 |
| Table 4.1 | Hardware and Software Resources | 21 |
| Table 4.2 | Test Plan and Verification | 22 |
| Table 4.3 | Experimental Results | 23 |
| Table 4.4 | Objective Achievement | 24 |
| Table 4.5 | Student Responsibilities | 25 |
| Table 4.6 | Project Budget | 26 |

# LIST OF ABBREVIATIONS / SYMBOLS

| Symbol | Description | Unit |
| --- | --- | --- |
| FFT | Fast Fourier Transform | — |
| IFFT | Inverse Fast Fourier Transform | — |
| FT | Fourier Transform | — |
| DSP | Digital Signal Processing | — |
| ADC | Analog-to-Digital Converter | — |
| DAC | Digital-to-Analog Converter | — |
| PCM | Pulse Code Modulation | — |
| Hz | Frequency | Hertz |
| kHz | Kilohertz | kHz |
| dB | Decibel | dB |
| SNR | Signal-to-Noise Ratio | dB |
| N | Number of Samples | — |
| Fs | Sampling Frequency | Hz |
| t | Time | s |
| f | Frequency | Hz |
| x[n] | Discrete-time input signal | — |
| X[k] | Frequency-domain representation | — |
| W | Window function | — |
| GPIO | General Purpose Input/Output | — |
| CPU | Central Processing Unit | — |
| RAM | Random Access Memory | — |
| I/O | Input/Output | — |

# CHAPTER 1: INTRODUCTION AND ENGINEERING PROBLEM

## 1.1 Background

Audio signals are an important form of information used in communication systems, voice recording, entertainment, online meetings, hearing assistance, surveillance, and human-computer interaction. However, recorded and transmitted audio signals are often contaminated by unwanted noise.

Noise can originate from several sources, including traffic, fans, air conditioners, electrical interference, machinery, surrounding conversations, and recording equipment. When noise is present at a significant level, it can reduce speech intelligibility and degrade the overall quality of the audio signal.

Traditional noise-reduction techniques may use simple analog filters or fixed digital filters. Although these methods can remove certain frequency components, they may not be effective when the noise characteristics vary with time or overlap with the useful audio spectrum.

Fourier Transform techniques provide an effective method for analyzing audio signals in terms of their frequency components. The Fast Fourier Transform (FFT) is an efficient algorithm for calculating the Discrete Fourier Transform (DFT). By converting an audio signal from the time domain into the frequency domain, the frequency components of both the desired audio and unwanted noise can be analyzed.

The proposed project applies FFT-based digital signal processing to reduce unwanted noise from an audio signal. The input audio is divided into manageable frames, transformed into the frequency domain using FFT, filtered to reduce unwanted frequency components, and converted back to the time domain using IFFT.

The system therefore follows a basic closed processing sequence:

Audio Input → Sampling → Preprocessing → FFT → Noise Identification → Frequency-Domain Filtering → IFFT → Denoised Audio Output

## 1.2 Need for the Project

Noise reduction is required in many practical audio applications because unwanted noise can interfere with useful information.

The major needs addressed by this project are:

- Reduction of unwanted background noise.

- Improvement of speech intelligibility.

- Improvement of recorded audio quality.

- Demonstration of Fourier Transform in a practical engineering application.

- Development of a real-time digital signal-processing system.

- Reduction of dependence on complex commercial audio-processing solutions.

- Development of an economical prototype for educational and experimental applications.

- Understanding of frequency-domain analysis and digital filtering.

The engineering significance of the project lies in the conversion of a real-world audio problem into a frequency-domain signal-processing problem.

## 1.3 Problem Statement

Audio recordings and communication signals are frequently contaminated by unwanted noise. Simple time-domain processing may not clearly distinguish useful audio information from unwanted frequency components.

Therefore, the engineering problem is to design and implement a real-time audio noise reduction system that can:

- acquire an audio signal,

- convert the signal into digital samples,

- analyze its frequency components using FFT,

- identify unwanted frequency components,

- attenuate the selected noise components,

- reconstruct the processed signal using IFFT,

- and produce a clearer audio output with acceptable processing delay.

The design must balance noise-reduction performance, audio quality, computational complexity, processing speed, memory requirements, and implementation cost.

## 1.4 Project Aim

The aim of this project is to design and develop a real-time audio noise reduction system using Fourier Transform techniques to identify and attenuate unwanted frequency components while preserving useful audio information.

## 1.5 Project Objectives

- To study the characteristics of noise present in audio signals.

- To understand the principles of Fourier Transform and FFT.

- To acquire and process digital audio signals.

- To transform audio signals from the time domain to the frequency domain.

- To identify unwanted frequency components using spectral analysis.

- To implement frequency-domain filtering for noise reduction.

- To reconstruct the processed audio using IFFT.

- To evaluate the quality of the denoised audio signal.

- To investigate the feasibility of real-time audio processing.

- To develop a low-cost and scalable prototype for audio noise reduction.

## 1.6 Scope

### Included

- Digital audio acquisition.

- Audio sampling.

- Signal preprocessing.

- FFT-based frequency analysis.

- Noise-frequency identification.

- Frequency-domain filtering.

- IFFT-based signal reconstruction.

- Real-time or near-real-time frame processing.

- Waveform and frequency-spectrum analysis.

- Comparison of noisy and processed audio.

- Performance evaluation.

### Excluded

- Professional studio-grade noise cancellation.

- Large-scale commercial audio processing.

- Complete AI-based speech enhancement.

- Hardware implementation of advanced adaptive noise cancellation.

- Automatic separation of multiple speakers.

- Perfect removal of non-stationary noise.

### Assumptions

The project assumes that:

- The audio input can be converted into a suitable digital format.

- The selected sampling frequency is sufficient for the audio bandwidth.

- The FFT size provides adequate frequency resolution.

- The selected filtering technique does not excessively distort useful audio.

- The processing hardware/software has sufficient computational capability for the selected frame size.

### Boundary Conditions

The prototype is primarily intended for educational, experimental, and small-scale real-time audio-processing applications.

## 1.7 Expected Engineering Outcome

The expected outcome is a functional real-time audio noise reduction system consisting of:

- Audio input source,

- Audio acquisition interface,

- Processing system,

- FFT algorithm,

- Noise identification method,

- Frequency-domain filter,

- IFFT reconstruction,

- Audio output.

The system should accept a noisy audio signal and generate a processed output with reduced unwanted frequency components.

# CHAPTER 2: LITERATURE REVIEW AND EXISTING SOLUTIONS

## 2.1 Review Method

The literature review focuses on digital signal processing, Fourier Transform, FFT algorithms, audio noise reduction, spectral filtering, speech enhancement, and real-time audio processing.

Existing research and engineering techniques were considered to understand how audio noise can be analyzed and reduced. The review particularly focuses on frequency-domain approaches because Fourier Transform techniques provide direct information about the spectral composition of audio signals.

## 2.2 Review of Existing Work

Existing audio noise-reduction systems can generally be classified into:

- Analog filtering.

- Digital fixed filtering.

- Spectral subtraction.

- Wiener filtering.

- Adaptive filtering.

- Fourier/FFT-based filtering.

- Machine-learning-based noise reduction.

Analog filtering can be effective for removing specific frequency components but provides limited flexibility.

Digital filters provide better programmability and can be designed for specific frequency ranges. However, fixed filters may not perform effectively when noise characteristics change.

Spectral subtraction estimates the noise spectrum and subtracts it from the noisy signal. It can provide useful noise reduction but may introduce musical noise under certain conditions.

Wiener filtering uses statistical information about the signal and noise to estimate an improved signal. It generally provides better performance but requires additional computation and assumptions about signal and noise characteristics.

Adaptive filtering continuously adjusts filter parameters based on the input signal. It is useful for changing noise conditions but requires a more complex implementation.

FFT-based filtering provides a convenient method of analyzing and manipulating the frequency components of an audio signal. It is therefore selected as the primary technique for this project.

## 2.3 Comparative Analysis

| Existing Approach | Advantages | Limitations | Relevance |
| --- | --- | --- | --- |
| Analog filtering | Simple hardware implementation | Limited flexibility | Provides basic filtering concept |
| Fixed digital filter | Easy to implement | Fixed response | Useful for selected noise frequencies |
| Spectral subtraction | Effective for stationary noise | May introduce artifacts | Possible enhancement |
| Wiener filter | Good noise reduction | Higher computational complexity | Advanced future option |
| Adaptive filtering | Handles changing noise | More complex algorithm | Future extension |
| FFT-based filtering | Frequency-domain control and efficient computation | Requires FFT/IFFT processing | Directly adopted |
| AI-based enhancement | Can handle complex noise | High computational and data requirements | Future extension |

Table 2.1: Comparison of Existing Audio Noise Reduction Approaches

## 2.4 Research / Engineering Gap

A major limitation of basic audio-processing systems is their inability to distinguish unwanted frequency components from useful audio in a flexible manner.

Simple fixed filters may remove noise but can also remove useful components if the noise overlaps the desired signal.

Advanced AI-based approaches can provide powerful noise reduction but often require large datasets, higher processing resources, and more complex implementation.

Therefore, there is a need for a practical intermediate solution that provides:

- frequency-domain analysis,

- programmable filtering,

- relatively low computational complexity,

- real-time processing capability,

- low implementation cost,

- and easy modification.

## 2.5 Proposed Contribution

The proposed project contributes a real-time audio noise reduction system based on Fourier Transform techniques.

The main contribution is the integration of:

Audio Acquisition + FFT Analysis + Noise Identification + Frequency-Domain Filtering + IFFT Reconstruction

The system provides a practical demonstration of how frequency-domain signal processing can be applied to a real-world audio engineering problem.

The architecture can also serve as a foundation for future integration of adaptive filters, spectral subtraction, machine-learning-based noise detection, and intelligent audio enhancement.

# CHAPTER 3: ENGINEERING DESIGN & TRADE-OFFS, SUSTAINABILITY, ETHICS, SAFETY, RISK & SECURITY

## 3.1 Requirements

### Stakeholder / User Requirements

| Stakeholder | Requirement |
| --- | --- |
| Audio user | Reduced background noise |
| User | Improved speech clarity |
| System developer | Easy implementation |
| System developer | Real-time processing |
| Researcher | Frequency-domain analysis |
| System operator | Reliable operation |
| Student | Low-cost prototype |
| Environment | Low computational and energy requirements |

Table 3.1: Stakeholder/User Requirements

### Functional & Non-Functional Requirements

| Requirement | Type | Measurable Target |
| --- | --- | --- |
| Acquire audio signal | Functional | Continuous audio input |
| Sample audio | Functional | Selected sampling frequency |
| Perform FFT | Functional | Real-time/near-real-time processing |
| Identify noise | Functional | Frequency-domain analysis |
| Filter unwanted components | Functional | Noise attenuation |
| Perform IFFT | Functional | Reconstructed output |
| Produce audio output | Functional | Audible processed signal |
| Low computational delay | Non-Functional | Suitable for real-time operation |
| Reliable operation | Non-Functional | Stable repeated processing |
| Low cost | Non-Functional | Student-project budget |
| Easy modification | Non-Functional | Modular software architecture |

Table 3.2: Functional and Non-Functional Requirements

## 3.2 Constraints & Applicable Standards

Project constraints include:

- Limited processing resources.

- Limited project budget.

- Sampling-frequency limitations.

- FFT computation requirements.

- Memory requirements.

- Noise characteristics varying with environment.

- Possible distortion caused by aggressive filtering.

- Real-time latency requirements.

- Audio input/output hardware limitations.

### Applicable Standards and Guidelines

| Standard / Guideline | Requirement | Application |
| --- | --- | --- |
| IEC 60268 series | Sound system equipment considerations | Relevant to audio equipment |
| IEC 61606 | Audio and audiovisual equipment measurement | Relevant to audio signal measurements |
| IEEE signal-processing practices | Digital signal-processing principles | Applied during algorithm development |
| PCM audio conventions | Digital audio representation | Used during audio acquisition/processing |
| Local electrical safety requirements | Safe electronic connections | Applied during hardware implementation |

Table 3.3: Applicable Standards and Guidelines

Standards should be verified against the exact hardware/software configuration before claiming formal compliance.

## 3.3 Design Specifications

| Parameter | Target / Requirement | Unit | Priority |
| --- | --- | --- | --- |
| Processing technique | FFT/IFFT | — | High |
| Input | Digital audio signal | — | High |
| Sampling frequency | Selected according to application | Hz | High |
| FFT size | Experimentally selected | Samples | High |
| Processing | Real-time/near-real-time | — | High |
| Filtering | Frequency-domain | — | High |
| Output | Denoised audio | — | High |
| Processing delay | Low | ms | High |
| System cost | Low | INR | Medium |
| Operation | Continuous/frame-based | — | High |

Table 3.4: Design Specifications

## 3.4 Alternative Solutions & Evaluation

### Alternative 1: Analog Filtering

Analog filters can be used before digital conversion to remove unwanted frequency components.

Advantage: Simple and fast.

Disadvantage: Limited flexibility and difficult to modify after hardware implementation.

### Alternative 2: Fixed Digital Filter

A digital low-pass, high-pass, or band-pass filter can be implemented using software.

Advantage: Easy to implement and computationally efficient.

Disadvantage: Fixed filtering may not adapt to changing noise.

### Alternative 3: FFT-Based Filtering

The audio signal is converted into the frequency domain using FFT, unwanted frequency components are attenuated, and the signal is reconstructed using IFFT.

Advantage: Provides direct frequency-domain control and is suitable for experimentation.

Disadvantage: Requires FFT/IFFT processing and careful filter selection.

### Alternative Evaluation

| Criterion | Weight | Analog | Fixed Digital | FFT-Based | AI-Based |
| --- | --- | --- | --- | --- | --- |
| Performance | 30% | 3 | 4 | 5 | 5 |
| Cost | 20% | 4 | 5 | 4 | 2 |
| Complexity | 15% | 4 | 5 | 4 | 2 |
| Flexibility | 20% | 2 | 3 | 5 | 5 |
| Real-time suitability | 15% | 5 | 5 | 4 | 2 |
| Overall | 100% | 3.55 | 4.35 | 4.55 | 3.25 |

Table 3.5: Alternative Solution Evaluation

The scores are engineering-assessment values for selecting the prototype approach and can be modified according to actual project requirements.

## 3.5 Selected Design & Detailed Design

The selected architecture consists of the following major subsystems:

Audio Input → Sampling → Preprocessing → FFT → Noise Detection → Frequency-Domain Filter → IFFT → Audio Output

### Stage 1: Audio Input

The input audio signal is obtained through a microphone, recorded audio file, or suitable audio-input interface.

### Stage 2: Sampling

The continuous audio signal is represented as discrete digital samples.

The sampling frequency is selected according to the required audio bandwidth.

According to the Nyquist principle:

Fs ≥ 2Fmax

where:

- Fs = sampling frequency

- Fmax = maximum frequency component to be represented.

### Stage 3: Preprocessing

The audio signal may be normalized and divided into frames before FFT processing.

A window function may also be applied to reduce spectral leakage.

### Stage 4: FFT

The FFT converts the time-domain audio signal into a frequency-domain representation.

The Discrete Fourier Transform is given by:

X[k] = Σ x[n]e^(-j2πkn/N)

where:

- x[n] = input audio samples,

- X[k] = frequency-domain samples,

- N = FFT size,

- k = frequency-bin index.

The FFT provides an efficient method for calculating this transformation.

### Stage 5: Noise Identification

The frequency spectrum is analyzed to identify frequency components associated with unwanted noise.

Depending on the noise type, the system can identify:

- low-frequency interference,

- high-frequency noise,

- narrow-band interference,

- periodic noise,

- or other unwanted spectral components.

### Stage 6: Frequency-Domain Filtering

Unwanted frequency components are attenuated using a selected frequency-domain filter.

The general filtering operation can be represented as:

Y[k] = H[k]X[k]

where:

- X[k] = input spectrum,

- H[k] = filter response,

- Y[k] = filtered spectrum.

### Stage 7: IFFT

The filtered frequency-domain signal is converted back into the time domain using the Inverse FFT.

### Stage 8: Audio Output

The reconstructed signal is played through an audio output device or saved as a processed audio file.

### Basic Processing Logic

Input Audio → FFT → Analyze Spectrum → Suppress Noise Frequencies → IFFT → Output Audio

## 3.6 Trade-off Analysis

| Design Decision | Alternative Considered | Benefit | Disadvantage | Final Decision & Justification |
| --- | --- | --- | --- | --- |
| FFT processing | Direct DFT | Faster computation | FFT implementation required | FFT selected |
| Frequency-domain filtering | Time-domain filtering | Direct frequency control | Requires FFT/IFFT | Frequency-domain filtering selected |
| Frame processing | Entire-file processing | Lower latency | Requires frame management | Frame processing selected |
| Fixed threshold | Adaptive threshold | Simple implementation | Less adaptive | Fixed threshold selected for prototype |
| Local processing | Cloud processing | Lower latency and cost | No remote processing | Local processing selected |
| Software implementation | Dedicated DSP hardware | Flexible and inexpensive | CPU limitations | Software prototype selected |

Table 3.6: Design Trade-off Analysis

## 3.7 Sustainability, Ethics, Safety, Risk & Security

| Domain | Consideration | Evidence Evaluated | Impact on Final Decision |
| --- | --- | --- | --- |
| Sustainability | Low computational resource usage | Local processing | Software-based solution selected |
| Sustainability | Reduced need for repeated recordings | Improved audio quality | Noise reduction adopted |
| Ethics | Responsible audio processing | Avoid unwanted modification | Moderate filtering selected |
| Privacy | Audio data protection | Local processing | No unnecessary cloud storage |
| Safety | Safe electronic connections | Hardware inspection | Proper power connections required |
| Security | Audio data protection | File/data access | Local data handling preferred |
| Reliability | Processing stability | Repeated tests | Stable frame processing required |

## 3.8 Project Risk Register

| Risk | Probability | Impact | Mitigation |
| --- | --- | --- | --- |
| Incorrect FFT implementation | Medium | High | Validate FFT using known signals |
| Excessive filtering | Medium | High | Use controlled frequency thresholds |
| Audio distortion | Medium | High | Compare input/output waveform |
| Processing delay | Medium | Medium | Optimize FFT size and frame length |
| Spectral leakage | Medium | Medium | Apply suitable windowing |
| Audio clipping | Medium | Medium | Normalize output signal |
| Hardware failure | Low | High | Check power and connections |
| Microphone interference | Medium | Medium | Test with different input sources |
| Software errors | Medium | Medium | Modular testing and debugging |
| Loss of audio data | Low | Medium | Maintain backup recordings |

Table 3.8: Project Risk Register

# CHAPTER 4: IMPLEMENTATION, TESTING & RESULTS, PROJECT MANAGEMENT

## 4.1 Development Approach & Resources

The project follows a systematic engineering development process consisting of:

- Problem identification.

- Literature review.

- Requirement analysis.

- Selection of Fourier Transform technique.

- Audio acquisition.

- Signal preprocessing.

- FFT implementation.

- Noise-frequency analysis.

- Frequency-domain filtering.

- IFFT reconstruction.

- Audio output.

- Testing and validation.

- Performance evaluation.

- Documentation.

The development process emphasizes modular testing so that each stage can be verified independently before complete system integration.

## 4.2 Hardware / Software / Fabrication Implementation & Modern Tools Used

### Hardware Implementation

Depending on the final prototype, the system can include:

- Computer/laptop or embedded processing platform.

- Microphone/audio input device.

- Audio output device.

- Audio interface where required.

- Suitable power supply.

- Connecting cables.

### Software Implementation

The software performs the following operations:

- Read audio input.

- Convert input into numerical samples.

- Normalize the signal.

- Divide the signal into frames.

- Apply a window function.

- Calculate FFT.

- Analyze the frequency spectrum.

- Identify unwanted frequency components.

- Apply frequency-domain filtering.

- Calculate IFFT.

- Reconstruct the output audio.

- Normalize the output.

- Play or save the denoised audio.

### Tools Used

| Tool / Software / Equipment | Purpose | Stage Used |
| --- | --- | --- |
| Python | Main programming environment | Development |
| NumPy | Numerical computation and FFT | Signal processing |
| SciPy | Signal processing functions | Filtering/analysis |
| Matplotlib | Waveform and spectrum visualization | Testing |
| OpenCV/Audio libraries where required | Input/output integration | Implementation |
| VS Code | Code development | Development |
| Microphone | Audio acquisition | Input |
| Speaker/Headphones | Audio output | Testing |
| Computer/Laptop | Processing | Implementation |

Table 4.1: Hardware and Software Resources

## 4.3 FFT-Based Algorithm

### Algorithm

Step 1: Acquire the noisy audio signal.

Step 2: Convert the signal into digital samples.

Step 3: Divide the signal into frames of N samples.

Step 4: Apply a suitable window function.

Step 5: Compute FFT.

Step 6: Calculate the magnitude spectrum.

Step 7: Identify frequency components corresponding to unwanted noise.

Step 8: Attenuate or remove the selected frequency components.

Step 9: Perform IFFT.

Step 10: Reconstruct the time-domain signal.

Step 11: Normalize the output to prevent clipping.

Step 12: Play or save the processed audio.

## 4.4 Test Plan & Verification

| Requirement | Test Method | Acceptance Criterion |
| --- | --- | --- |
| Audio acquisition | Provide audio input | Audio samples obtained |
| FFT processing | Process known test signal | Expected spectrum obtained |
| Frequency identification | Analyze noisy signal | Noise components identified |
| Filtering | Apply frequency-domain filter | Unwanted components attenuated |
| IFFT reconstruction | Convert processed spectrum | Time-domain signal reconstructed |
| Audio output | Play/save processed signal | Output available |
| Repeated processing | Run multiple samples | Stable operation |
| Real-time processing | Process continuous frames | Acceptable latency |
| Signal quality | Compare input/output | Improved noise condition |
| System stability | Long-duration operation | No unexpected failure |

Table 4.2: Test Plan and Verification

# 4.5 Experimental Testing

The system should be tested using different audio conditions.

### Test 1: Clean Audio

A relatively clean audio signal is processed to verify that the system does not unnecessarily modify useful audio.

### Test 2: Audio with Background Noise

Speech/audio is combined with background noise and supplied to the system.

### Test 3: Low-Level Noise

A small amount of background noise is introduced to determine whether the system can perform noise reduction without excessive signal distortion.

### Test 4: High-Level Noise

A higher noise level is introduced to evaluate the limits of the filtering technique.

### Test 5: Different Noise Frequencies

Audio containing different frequency components is tested to determine how effectively the FFT-based filter handles different noise conditions.

# 4.6 Results

The following table can be completed using your actual experimental measurements.

| Test Condition | Noise Condition | Expected Result | Observed Result | Status |
| --- | --- | --- | --- | --- |
| Test 1 | Clean audio | Audio preserved | [TO BE FILLED] | [PASS/FAIL] |
| Test 2 | Low background noise | Noise reduced | [TO BE FILLED] | [PASS/FAIL] |
| Test 3 | Moderate background noise | Noticeable noise reduction | [TO BE FILLED] | [PASS/FAIL] |
| Test 4 | High background noise | Noise attenuation | [TO BE FILLED] | [PASS/FAIL] |
| Test 5 | Narrow-band noise | Selected frequencies reduced | [TO BE FILLED] | [PASS/FAIL] |

Table 4.3: Experimental Results

## 4.7 Data Analysis & Engineering Judgment

The experimental data should be analyzed by comparing the noisy input signal with the processed output signal.

The time-domain waveform can be used to observe changes in signal amplitude, while the frequency spectrum can be used to determine whether unwanted frequency components have been reduced.

The magnitude spectrum is particularly important because the project is based on Fourier Transform techniques. A successful filtering operation should result in attenuation of the targeted noise components while retaining the major frequency components associated with the desired audio.

One possible performance measure is the Signal-to-Noise Ratio (SNR).

The SNR can be represented as:

SNR = 10 log₁₀(Psignal / Pnoise)

where:

- Psignal = signal power,

- Pnoise = noise power.

An improvement in SNR after processing indicates that the relative contribution of noise has been reduced.

Other evaluation parameters can include:

- Processing time.

- Latency.

- Noise attenuation.

- Frequency-spectrum reduction.

- Output amplitude.

- Audio distortion.

- Subjective listening quality.

The actual values should be calculated from the experimental data obtained during testing.

## 4.8 Comparison with Existing Solutions & Achievement of Objectives

| Objective | Evidence | Achievement |
| --- | --- | --- |
| Study audio noise | Literature and signal analysis | Achieved |
| Understand Fourier Transform | FFT implementation | Achieved |
| Develop audio acquisition | Input audio system | Achieved |
| Implement FFT | FFT processing module | Achieved |
| Identify noise frequencies | Spectrum analysis | Achieved |
| Implement filtering | Frequency-domain filter | Achieved |
| Reconstruct audio | IFFT implementation | Achieved |
| Test different noise conditions | Experimental testing | [TO BE FILLED] |
| Evaluate performance | SNR/spectral analysis | [TO BE FILLED] |
| Develop real-time prototype | Continuous/frame processing | [TO BE FILLED] |

Table 4.4: Objective Achievement

# 4.9 Strengths & Limitations

## Strengths

- Real-time or near-real-time processing capability.

- Uses Fourier Transform for practical signal analysis.

- Frequency-domain noise identification.

- Programmable filtering.

- Low-cost software implementation.

- Easy to modify.

- Suitable for educational DSP applications.

- Provides visual waveform and spectrum analysis.

- Can be extended to advanced filtering techniques.

- Can be integrated with embedded systems in future.

## Limitations

- Performance depends on the type of noise.

- Noise overlapping with useful audio frequencies may be difficult to remove.

- Aggressive filtering may distort the desired audio.

- FFT size affects frequency resolution and processing delay.

- Spectral leakage can affect frequency identification.

- Real-time performance depends on available processing resources.

- Basic threshold-based filtering may not adapt automatically to changing noise.

- A single filtering strategy may not work equally well for all noise environments.

# 4.10 Project Planning, Roles & Budget

## Project Schedule

| Phase | Activity |
| --- | --- |
| Week 1 | Problem identification and literature review |
| Week 2 | Requirement analysis |
| Week 3 | Study of Fourier Transform and FFT |
| Week 4 | Audio acquisition setup |
| Week 5 | FFT programming |
| Week 6 | Noise-frequency analysis |
| Week 7 | Frequency-domain filter implementation |
| Week 8 | IFFT and audio reconstruction |
| Week 9 | Testing and results analysis |
| Week 10 | Report preparation and final demonstration |

## Student Responsibilities

| Student | Assigned Responsibility | Actual Contribution |
| --- | --- | --- |
| Student 1 | Audio acquisition and preprocessing | Audio input setup, sampling and preprocessing |
| Student 2 | FFT and filtering | FFT implementation, frequency analysis and filtering |
| Student 3 | System integration and testing | Complete integration, testing, analysis and documentation |

Table 4.5: Student Responsibilities

Table 4.6: Project Budget

# CHAPTER 5: CONCLUSION, FUTURE WORK AND REFLECTION

## 5.1 Conclusion

The Real-Time Audio Noise Reduction Using Fourier Transform Techniques project was developed to address the engineering problem of unwanted noise in audio signals.

The proposed system uses digital signal-processing techniques to analyze and process audio in the frequency domain. The input audio signal is converted into digital samples and divided into frames for processing. FFT is then used to transform the signal from the time domain to the frequency domain, where unwanted frequency components can be identified and attenuated.

After frequency-domain filtering, the IFFT is used to reconstruct the processed audio signal in the time domain. The resulting output can then be compared with the original noisy signal using waveform and frequency-spectrum analysis.

The project demonstrates the practical application of Fourier Transform concepts in digital audio processing. It provides a relatively simple and economical approach to noise reduction while maintaining flexibility for future improvements.

The developed system also demonstrates the importance of balancing noise reduction and preservation of useful audio information. Excessive filtering can reduce noise but may also remove important audio components. Therefore, appropriate frequency selection, FFT size, windowing, and filtering parameters are important engineering considerations.

Overall, the project establishes a practical foundation for real-time audio enhancement and demonstrates how mathematical signal-processing techniques can be converted into an engineering application.

# 5.2 Future Work

The following improvements can be implemented in future versions:

- Implementation of adaptive noise filtering.

- Integration of spectral subtraction.

- Implementation of Wiener filtering.

- Automatic noise-frequency detection.

- Machine-learning-based noise classification.

- Deep-learning-based speech enhancement.

- Real-time microphone-to-speaker hardware implementation.

- Integration with ESP32 or other embedded processors where computational requirements permit.

- Development of a graphical user interface.

- Addition of adjustable noise-reduction levels.

- Support for multiple audio channels.

- Development of mobile-based audio processing.

- Cloud-based audio analysis where privacy requirements permit.

- Automatic SNR measurement.

- Development of a low-latency dedicated DSP implementation.

- Integration with communication systems.

- Development of an intelligent system capable of adapting to changing noise environments.

# 5.3 Professional Learning & Individual Reflection

## New Knowledge Acquired

Through this project, the following technical knowledge was developed:

- Fourier Transform principles.

- Fast Fourier Transform implementation.

- Inverse Fourier Transform.

- Digital signal processing.

- Audio sampling.

- Frequency-domain analysis.

- Digital filtering.

- Noise characterization.

- Signal reconstruction.

- Waveform visualization.

- Spectrum analysis.

- Real-time signal processing.

## Engineering & Professional Skills Developed

The project helped develop:

- Programming skills.

- Algorithm development.

- Signal-processing skills.

- System design.

- Debugging.

- Experimental testing.

- Data analysis.

- Technical documentation.

- Teamwork.

- Project management.

- Engineering decision-making.

# Student 1 – Individual Reflection

The major challenge was acquiring and preprocessing the audio signal in a form suitable for digital signal processing. Audio signals contain a wide range of frequency components, and inappropriate sampling or preprocessing can affect the quality of FFT analysis.

This challenge was addressed by studying audio sampling, signal normalization, frame-based processing, and frequency-domain representation. A key engineering decision was to use FFT because it provides an efficient method for analyzing the frequency components of an audio signal.

The project improved my understanding of audio acquisition, digital sampling, and preprocessing. If the project were redesigned, multiple audio-input conditions would be tested to improve the reliability of the system.

# Student 2 – Individual Reflection

The main engineering challenge was implementing the FFT-based frequency analysis and selecting appropriate noise-filtering parameters. Removing too many frequency components can affect the desired audio signal, while insufficient filtering may leave significant background noise.

The problem was addressed by analyzing the magnitude spectrum and selecting frequency regions associated with unwanted noise. The use of FFT provided a practical method for observing these frequency components.

The project helped improve my understanding of Fourier Transform, FFT algorithms, digital filtering, and frequency-domain signal processing. In a future version, adaptive filtering or machine-learning-based noise identification could be implemented.

# Student 3 – Individual Reflection

The major challenge was integrating the complete audio-processing pipeline into a reliable system. The audio input, FFT processing, filtering, IFFT reconstruction, and output stages had to operate together without significant distortion or processing delay.

This challenge was addressed through subsystem-level testing and gradual integration. Testing individual processing stages helped identify errors before complete system implementation.

# REFERENCES

The following references can be used as the initial reference list for the report. Verify the exact edition, year, volume, pages, and DOI against the source you actually use before final submission.

[1] A. V. Oppenheim and R. W. Schafer, Discrete-Time Signal Processing, 3rd ed., Pearson, 2010.

[2] R. G. Lyons, Understanding Digital Signal Processing, 3rd ed., Pearson, 2011.

[3] J. G. Proakis and D. G. Manolakis, Digital Signal Processing: Principles, Algorithms, and Applications, 4th ed., Pearson, 2007.

[4] S. W. Smith, The Scientist and Engineer's Guide to Digital Signal Processing, California Technical Publishing, 1997.

[5] A. V. Oppenheim, R. W. Schafer, and J. R. Buck, Discrete-Time Signal Processing, 2nd ed., Prentice Hall, 1999.

[6] L. R. Rabiner and R. W. Schafer, Theory and Applications of Digital Speech Processing, Pearson, 2011.

[7] P. Vary and R. Martin, Digital Speech Transmission: Enhancement, Coding and Error Concealment, Wiley, 2006.

[8] Y. Ephraim and D. Malah, “Speech enhancement using a minimum mean-square error short-time spectral amplitude estimator,” IEEE Transactions on Acoustics, Speech, and Signal Processing, vol. 32, no. 6, pp. 1109–1121, 1984.

[9] Y. Ephraim and D. Malah, “Speech enhancement using a minimum mean-square error log-spectral amplitude estimator,” IEEE Transactions on Acoustics, Speech, and Signal Processing, vol. 33, no. 2, pp. 443–445, 1985.

[10] J. Allen, “Short term spectral analysis, synthesis, and modification by discrete Fourier transform,” IEEE Transactions on Acoustics, Speech, and Signal Processing, vol. 25, no. 3, pp. 235–238, 1977.

[11] J. W. Cooley and J. W. Tukey, “An algorithm for the machine calculation of complex Fourier series,” Mathematics of Computation, vol. 19, no. 90, pp. 297–301, 1965.

[12] A. V. Oppenheim and R. W. Schafer, Digital Signal Processing, Prentice Hall.

[13] NumPy Developers, NumPy Documentation: Discrete Fourier Transform, NumPy Documentation.

[14] SciPy Developers, SciPy Signal Processing Documentation, SciPy Documentation.

[15] Python Software Foundation, Python Documentation, Python Documentation.

[16] International Electrotechnical Commission, relevant IEC standards for audio and audiovisual equipment and measurement.

[17] IEEE, relevant standards and technical literature related to digital signal processing and audio engineering.

[18] R. Martin, “Noise power spectral density estimation based on optimal smoothing and minimum statistics,” IEEE Transactions on Speech and Audio Processing, vol. 9, no. 5, pp. 504–512, 2001.

[19] P. C. Loizou, Speech Enhancement: Theory and Practice, 2nd ed., CRC Press, 2013.

[20] T. F. Quatieri, Discrete-Time Speech Signal Processing: Principles and Practice, Pearson, 2002.